# Experiment-6

AIM: Perform FFT and IFFT on discrete time signal.

Theory:

The Fast Fourier Transform (FFT) is an algorithm used to efficiently compute the Discrete Fourier Transform (DFT) of a sequence or signal. It converts a time-domain signal into its frequency-domain representation, revealing the signal's frequency components.

The Inverse Fast Fourier Transform (IFFT) is the reverse process of the FFT. It transforms the frequency-domain representation back into the time-domain signal.

The FFT and IFFT are widely used in various applications, including signal processing, image processing, audio processing, and communication systems.

Program:

import numpy as np

import matplotlib.pyplot as plt

signal = np.array([1, 2, 3, 4, 5, 6, 7, 8])

# Compute the FFT of the signal

fft_result = np.fft.fft(signal)

# Compute the magnitude and Phase spectrum of the FFT result

magnitude_spectrum = np.abs(fft_result)

phase_spectrum = np.angle(fft_result)

# Compute the IFFT of the FFT result

reconstructed_signal = np.fft.ifft(fft_result)

# Create frequency axis for plotting

N = len(signal)

frequencies = np.arange(N)

# Plot the results in a 2x2 subplot

plt.figure(figsize=(12, 10))

# Original Signal

plt.subplot(2, 2, 1)

plt.stem(frequencies, signal, basefmt='b-')

plt.title('Original Signal')

plt.xlabel('Sample Index')

plt.ylabel('Amplitude')

plt.grid(True)

# Magnitude Spectrum

plt.subplot(2, 2, 2)

plt.stem(frequencies, magnitude_spectrum, basefmt='r-')

plt.title('Magnitude Spectrum')

plt.xlabel('Frequency Bin')

plt.ylabel('Magnitude')

plt.grid(True)

# Phase Spectrum

plt.subplot(2, 2, 3)

plt.stem(frequencies, phase_spectrum, basefmt='g-')

plt.title('Phase Spectrum')

plt.xlabel('Frequency Bin')

plt.ylabel('Phase (radians)')

plt.grid(True)

# Reconstructed Signal (Real part)

plt.subplot(2, 2, 4)

plt.stem(frequencies, np.real(reconstructed_signal), basefmt='m-')

plt.title('Reconstructed Signal (IFFT)')

plt.xlabel('Sample Index')

plt.ylabel('Amplitude')

plt.grid(True)

plt.tight_layout()

plt.show()

# Display numerical results

print("Original Signal:", signal)

print("FFT Result:", fft_result)

print("Magnitude Spectrum:", magnitude_spectrum)

print("Phase Spectrum:", phase_spectrum)

print("Reconstructed Signal (Real):", np.real(reconstructed_signal))

print("Reconstruction Error:", np.sum(np.abs(signal - np.real(reconstructed_signal))))

Output:

Conclusion:

- The FFT analysis demonstrates perfect signal reconstruction through the FFT-IFFT process.

- The magnitude spectrum shows the frequency content of the signal.

- The phase spectrum indicates the phase relationships between frequency components.

- The reconstructed signal matches the original, confirming the reversibility of the FFT operation.