# Experiment-4

Name: Milan Jani

Enrollment number: 92301733041

AIM: Design Butterworth and Chebyshev filter using bilinear transformation method.

Theory: The bilinear transformation method is commonly used to design analog filters and then convert them into digital filters. This method maps the analog frequency response to the digital frequency response using a bilinear transformation.

The Butterworth and Chebyshev filters are two commonly used filter types. The Butterworth filter has a maximally flat frequency response in the passband, while the Chebyshev filter allows for a sharper transition between the passband and the stopband at the expense of ripples in either the passband or stopband.

Program:

import numpy as np

import matplotlib.pyplot as plt

from scipy.signal import butter, bilinear, freqz, cheby1

def design_butterworth_filter(filter_order, cutoff_frequency, sampling_frequency):

# Design the analog Butterworth filter

analog_b, analog_a = butter(filter_order, cutoff_frequency, analog=True, btype='low')

# Perform the bilinear transformation

digital_b, digital_a = bilinear(analog_b, analog_a, sampling_frequency)

return digital_b, digital_a

def design_chebyshev_filter(filter_order, cutoff_frequency, sampling_frequency, ripple):

# Design the analog Chebyshev filter

analog_b, analog_a = cheby1(filter_order, ripple, cutoff_frequency, analog=True, btype='low')

# Perform the bilinear transformation

digital_b, digital_a = bilinear(analog_b, analog_a, sampling_frequency)

return digital_b, digital_a

def plot_filter_response(butter_b, butter_a, cheby_b, cheby_a, sampling_frequency):

# Create a single figure with 2x2 subplots

plt.figure(figsize=(15, 10))

# Butterworth Filter - Magnitude Response

frequency_b, magnitude_response_b = freqz(butter_b, butter_a, fs=sampling_frequency)

plt.subplot(2, 2, 1)

plt.plot(frequency_b, np.abs(magnitude_response_b))

plt.title('Butterworth Filter Magnitude Response')

plt.xlabel('Frequency (Hz)')

plt.ylabel('Magnitude')

plt.grid(True)

# Butterworth Filter - Impulse Response

_, impulse_response_b = freqz(butter_b, butter_a, fs=sampling_frequency, worN=512)

plt.subplot(2, 2, 2)

plt.plot(np.real(impulse_response_b))

plt.title('Butterworth Filter Impulse Response')

plt.xlabel('Samples')

plt.ylabel('Amplitude')

plt.grid(True)

# Chebyshev Filter - Magnitude Response

frequency_c, magnitude_response_c = freqz(cheby_b, cheby_a, fs=sampling_frequency)

plt.subplot(2, 2, 3)

plt.plot(frequency_c, np.abs(magnitude_response_c))

plt.title('Chebyshev Filter Magnitude Response')

plt.xlabel('Frequency (Hz)')

plt.ylabel('Magnitude')

plt.grid(True)

# Chebyshev Filter - Impulse Response

_, impulse_response_c = freqz(cheby_b, cheby_a, fs=sampling_frequency, worN=512)

plt.subplot(2, 2, 4)

plt.plot(np.real(impulse_response_c))

plt.title('Chebyshev Filter Impulse Response')

plt.xlabel('Samples')

plt.ylabel('Amplitude')

plt.grid(True)

plt.tight_layout()

plt.show()

# Specify the desired filter specifications

filter_order = 4  # Filter order

cutoff_frequency = 1000  # Cutoff frequency in Hz

sampling_frequency = 8000  # Sampling frequency in Hz

ripple = 0.5  # Ripple factor for Chebyshev filter

# Design the Butterworth filter

butter_b, butter_a = design_butterworth_filter(filter_order, cutoff_frequency, sampling_frequency)

# Design the Chebyshev filter

cheby_b, cheby_a = design_chebyshev_filter(filter_order, cutoff_frequency, sampling_frequency, ripple)

# Plot both filters' responses in a single 2x2 subplot

plot_filter_response(butter_b, butter_a, cheby_b, cheby_a, sampling_frequency)

# Save the filter coefficients (optional)

filter_path = 'filter_coefficients.txt'

np.savetxt(filter_path, np.vstack((cheby_b, cheby_a)), delimiter=',')

print(f"Filter coefficients saved at: {filter_path}")

Output:

Conclusion:

- Butterworth filter: Smooth rolloff, no ripples, better phase response

- Chebyshev filter: Sharp cutoff, faster rolloff, but has passband ripples

- Both filters are stable with decaying impulse responses

- Choice depends on requirement: flat response (Butterworth) vs sharp cutoff (Chebyshev)