AIM: To simulate and analyze fundamental signals such as unit impulse, impulse train, unit step, and ramp signals in both continuous and discrete time domains, and to understand their properties, behavior, and importance in digital signal processing.

- Unit impulse signal:-

import numpy as np

import matplotlib.pyplot as plt

def unit_impulse(length, position):

signal = np.zeros(length)

signal[position] = 1

return signal

# Parameters

start = -10  # Start value of the x-axis range

stop = 10  # Stop value of the x-axis range

step = 1  # Step size

# Generate x-axis values

x = np.arange(start, stop+step, step)

# Generate unit impulse signal

impulse_signal = unit_impulse(len(x), abs(start)//step)

# Plot the signal

plt.stem(x, impulse_signal)

plt.xlabel('Time')

plt.ylabel('Amplitude')

plt.title('Unit Impulse Signal')

plt.grid(True)

plt.show()

Output:-

- Impulse train signal:-

import numpy as np

import matplotlib.pyplot as plt

def simulate_impulse_train(signal_length, period):

impulse_train = np.zeros(signal_length)

for n in range(signal_length):

if n % period == 0:

impulse_train[n] = 1

return impulse_train

# Define the parameters for the impulse train

signal_length = 100  # Length of the impulse train

period = 10  # Period of the impulse train

# Simulate the impulse train

impulse_train = simulate_impulse_train(signal_length, period)

# Plot and display the impulse train

plt.stem(impulse_train)

plt.title('Impulse Train')

plt.xlabel('Sample')

plt.ylabel('Amplitude')

plt.show()

# Save the impulse train array (optional)

# np.savetxt('impulse_train.txt', impulse_train, delimiter=',')

Output:-

- Continuous and Discrete unit step signal’s:-

import numpy as np

import matplotlib.pyplot as plt

def simulate_continuous_unit_step(time):

unit_step = np.zeros_like(time)

unit_step[time >= 0] = 1

return unit_step

def simulate_discrete_unit_step(num_samples):

unit_step = np.zeros(num_samples)

unit_step[num_samples // 2:] = 1

return unit_step

# Define the time range for the continuous unit step signal

time = np.linspace(-5, 5, 1000)  # Time range from -5 to 5

# Simulate the continuous unit step signal

continuous_unit_step = simulate_continuous_unit_step(time)

# Define the number of samples for the discrete unit step signal

num_samples = 20  # Number of samples

# Simulate the discrete unit step signal

discrete_unit_step = simulate_discrete_unit_step(num_samples)

# Plot and display the continuous and discrete unit step signals

plt.figure(figsize=(10, 6))

plt.subplot(2, 1, 1)

plt.plot(time, continuous_unit_step)

plt.title('Continuous Unit Step Signal')

plt.xlabel('Time')

plt.ylabel('Amplitude')

plt.subplot(2, 1, 2)

plt.stem(discrete_unit_step)

plt.title('Discrete Unit Step Signal')

plt.xlabel('Sample')

plt.ylabel('Amplitude')

plt.tight_layout()

plt.show()

# Save the unit step signal arrays (optional)

# np.savetxt('continuous_unit_step.txt', continuous_unit_step, delimiter=',')

# np.savetxt('discrete_unit_step.txt', discrete_unit_step, delimiter=',')

Output:-

- Continuous and Discrete ramp signal:-

import numpy as np

import matplotlib.pyplot as plt

def simulate_continuous_ramp(time, slope):

ramp = np.zeros_like(time)

ramp[time >= 0] = slope * time[time >= 0]

return ramp

def simulate_discrete_ramp(num_samples, slope):

ramp = np.zeros(num_samples)

ramp[num_samples // 2:] = slope * np.arange(num_samples // 2, num_samples)

return ramp

# Define the time range for the continuous ramp signal

time = np.linspace(-5, 5, 1000)  # Time range from -5 to 5

# Define the number of samples and slope for the discrete ramp signal

num_samples = 20  # Number of samples

slope = 2  # Slope of the ramp

# Simulate the continuous ramp signal

continuous_ramp = simulate_continuous_ramp(time, slope)

# Simulate the discrete ramp signal

discrete_ramp = simulate_discrete_ramp(num_samples, slope)

# Plot and display the continuous and discrete ramp signals

plt.figure(figsize=(10, 6))

plt.subplot(2, 1, 1)

plt.plot(time, continuous_ramp)

plt.title('Continuous Ramp Signal')

plt.xlabel('Time')

plt.ylabel('Amplitude')

plt.subplot(2, 1, 2)

plt.stem(discrete_ramp)

plt.title('Discrete Ramp Signal')

plt.xlabel('Sample')

plt.ylabel('Amplitude')

plt.tight_layout()

plt.show()

# Save the ramp signal arrays (optional)

# np.savetxt('continuous_ramp.txt', continuous_ramp, delimiter=',')

# np.savetxt('discrete_ramp.txt', discrete_ramp, delimiter=',')

OUTPUT:-

- Continuous and Discrete exponential signal’s:-

import numpy as np

import matplotlib.pyplot as plt

def simulate_continuous_exponential(time, amplitude, coefficient):

exponential_signal = amplitude * np.exp(coefficient * time)

return exponential_signal

def simulate_discrete_exponential(num_samples, amplitude, coefficient):

exponential_signal = amplitude * np.exp(coefficient * np.arange(num_samples))

return exponential_signal

# Define the time range for the continuous exponential signal

time = np.linspace(0, 5, 1000)  # Time range from 0 to 5

# Define the number of samples, initial amplitude, and coefficient for the discrete exponential signal

num_samples = 20  # Number of samples

amplitude = 2  # Initial amplitude

coefficient = -0.5  # Exponential coefficient

# Simulate the continuous exponential signal

continuous_exponential = simulate_continuous_exponential(time, amplitude, coefficient)

# Simulate the discrete exponential signal

discrete_exponential = simulate_discrete_exponential(num_samples, amplitude, coefficient)

# Plot and display the continuous and discrete exponential signals

plt.figure(figsize=(10, 6))

plt.subplot(2, 1, 1)

plt.plot(time, continuous_exponential)

plt.title('Continuous Exponential Signal')

plt.xlabel('Time')

plt.ylabel('Amplitude')

plt.subplot(2, 1, 2)

plt.stem(discrete_exponential)

plt.title('Discrete Exponential Signal')

plt.xlabel('Sample')

plt.ylabel('Amplitude')

plt.tight_layout()

plt.show()

# Save the exponential signal arrays (optional)

# np.savetxt('continuous_exponential.txt', continuous_exponential, delimiter=',')

# np.savetxt('discrete_exponential.txt', discrete_exponential, delimiter=',')

Output:-

- Continuous and Discrete parabolic signal’s:-

import numpy as np

import matplotlib.pyplot as plt

def simulate_continuous_parabolic(time, coefficients):

parabolic_signal = np.polyval(coefficients, time)

return parabolic_signal

def simulate_discrete_parabolic(num_samples, coefficients):

parabolic_signal = np.polyval(coefficients, np.arange(num_samples))

return parabolic_signal

# Define the time range for the continuous parabolic signal

time = np.linspace(-5, 5, 1000)  # Time range from -5 to 5

# Define the number of samples and coefficients for the discrete parabolic signal

num_samples = 20  # Number of samples

coefficients = [1, 2, 1]  # Coefficients of the parabolic signal

# Simulate the continuous parabolic signal

continuous_parabolic = simulate_continuous_parabolic(time, coefficients)

# Simulate the discrete parabolic signal

discrete_parabolic = simulate_discrete_parabolic(num_samples, coefficients)

# Plot and display the continuous and discrete parabolic signals

plt.figure(figsize=(10, 6))

plt.subplot(2, 1, 1)

plt.plot(time, continuous_parabolic)

plt.title('Continuous Parabolic Signal')

plt.xlabel('Time')

plt.ylabel('Amplitude')

plt.subplot(2, 1, 2)

plt.stem(discrete_parabolic)

plt.title('Discrete Parabolic Signal')

plt.xlabel('Sample')

plt.ylabel('Amplitude')

plt.tight_layout()

plt.show()

# Save the parabolic signal arrays (optional)

# np.savetxt('continuous_parabolic.txt', continuous_parabolic, delimiter=',')

# np.savetxt('discrete_parabolic.txt', discrete_parabolic, delimiter=',')

Output:-

- Continuous and Discrete sine wave signal:-

import numpy as np

import matplotlib.pyplot as plt

def simulate_continuous_sine_wave(time, amplitude, frequency, phase):

sine_wave = amplitude * np.sin(2 * np.pi * frequency * time + phase)

return sine_wave

def simulate_discrete_sine_wave(num_samples, sampling_frequency, amplitude, frequency, phase):

time = np.arange(num_samples) / sampling_frequency

sine_wave = amplitude * np.sin(2 * np.pi * frequency * time + phase)

return sine_wave

# Define the time range for the continuous sine wave signal

time = np.linspace(0, 1, 1000)  # Time range from 0 to 1 second

# Define the number of samples, sampling frequency, and parameters for the discrete sine wave signal

num_samples = 100  # Number of samples

sampling_frequency = 10  # Sampling frequency in Hz

amplitude = 1  # Amplitude of the sine wave

frequency = 2  # Frequency of the sine wave in Hz

phase = 0  # Phase angle of the sine wave in radians

# Simulate the continuous sine wave signal

continuous_sine_wave = simulate_continuous_sine_wave(time, amplitude, frequency, phase)

# Simulate the discrete sine wave signal

discrete_sine_wave = simulate_discrete_sine_wave(num_samples, sampling_frequency, amplitude, frequency, phase)

# Plot and display the continuous and discrete sine wave signals

plt.figure(figsize=(10, 6))

plt.subplot(2, 1, 1)

plt.plot(time, continuous_sine_wave)

plt.title('Continuous Sine Wave Signal')

plt.xlabel('Time (s)')

plt.ylabel('Amplitude')

plt.subplot(2, 1, 2)

plt.stem(discrete_sine_wave)

plt.title('Discrete Sine Wave Signal')

plt.xlabel('Sample')

plt.ylabel('Amplitude')

plt.tight_layout()

plt.show()

# Save the sine wave signal arrays (optional)

# np.savetxt('continuous_sine_wave.txt', continuous_sine_wave, delimiter=',')

# np.savetxt('discrete_sine_wave.txt', discrete_sine_wave, delimiter=',')

Output:-

- Simulating the signal of given function  y(t)=u(t)+u(t-1)+3u(t+5).

import numpy as np

import matplotlib.pyplot as plt

def simulate_function(time):

y = np.zeros_like(time)

y[time >= 0] = 1

y[time >= 1] += 1

y[time >= -5] += 3

return y

# Define the time range

time = np.linspace(-10, 10, 1000)

# Simulate the function

function_values = simulate_function(time)

# Plot and display the function

plt.plot(time, function_values)

plt.title('Function y(t) = u(t) + u(t-1) + 3*u(t+5)')

plt.xlabel('Time')

plt.ylabel('Amplitude')

plt.ylim([-0.5, 5.5])

plt.grid(True)

plt.show()

Output:-

- Simulate given function  y(t)=Delta(t)+delta(t-1)+3*delta(t+5).

import numpy as np

import matplotlib.pyplot as plt

def simulate_function(time):

y = np.zeros_like(time)

y[time == 0] = 1

y[time == 1] += 1

y[time == -5] += 3

return y

# Define the time range

time = np.arange(-10, 11)

# Simulate the function

function_values = simulate_function(time)

# Plot and display the function

plt.stem(time, function_values)

plt.title('Function y(t) = Delta(t) + delta(t-1) + 3*delta(t+5)')

plt.xlabel('Time')

plt.ylabel('Amplitude')

plt.ylim([-0.5, 4.5])

plt.grid(True)

plt.show()

Output:-

Conclusion:-

In this experiment, we created and studied some basic signals used in signal processing, such as unit impulse, impulse train, unit step, and ramp signals, in both continuous and discrete forms. By changing their parameters and observing their behavior, we understood their mathematical properties, how they change over time, and why they are important in analyzing and designng systems. This hands-on work made the theory clearer and showed how these basic signals act as the foundation for more advanced signal processing.