# Experiment-2-Open_handed

AIM: Simulate linear convolution and circular convolution on discrete time signals.

# Code:

# from pydub import AudioSegment

# import numpy as np

# import matplotlib.pyplot as plt

# from scipy.signal import convolve, deconvolve, wiener

# from scipy.fft import fft, ifft

# # Load MP3 file

# audio = AudioSegment.from_mp3("DSIP/song.mp3")

# audio = audio.set_channels(1)

# samples = np.array(audio.get_array_of_samples()).astype(np.float32)

# sample_rate = audio.frame_rate

# # Normalize original samples

# samples_norm = samples / np.max(np.abs(samples))

# # Impulse Response 1: Simple Echo Suppression

# ir1 = np.array([1, 0, -0.5, 0, 0.25], dtype=np.float32)

# # Impulse Response 2: High-Pass Filter (removes low-freq bass/background music)

# ir2 = np.array([-0.1, -0.2, 1.0, -0.2, -0.1], dtype=np.float32)

# # Impulse Response 3: Notch Filter (suppresses specific frequency)

# ir3 = np.array([0.2, -0.3, 1.0, -0.3, 0.2], dtype=np.float32)

# # Impulse Response 4: Comb Filter (removes periodic patterns)

# ir4_length = 100

# ir4 = np.zeros(ir4_length, dtype=np.float32)

# ir4[0] = 1.0

# ir4[50] = -0.6  # Delay of 50 samples

# # Impulse Response 5: Low-Pass Suppression

# ir5 = np.array([0.05, 0.1, 0.7, 0.1, 0.05], dtype=np.float32)

# impulse_responses = {

# 'Echo Suppression': ir1,

# 'High-Pass Filter': ir2,

# 'Notch Filter': ir3,

# 'Comb Filter': ir4,

# 'Low-Pass Suppression': ir5

# }

# # Apply Each Impulse Response

# results = {}

# for name, ir in impulse_responses.items():

# convoluted = convolve(samples_norm, ir, mode='same')

# results[name] = convoluted / np.max(np.abs(convoluted))

# # Inverse Filtering (Deconvolution)

# try:

# H = fft(ir1, n=len(samples_norm))

# Y = fft(results['Echo Suppression'])

# epsilon = 1e-10

# X_recovered = Y / (H + epsilon)

# inverse_filtered = np.real(ifft(X_recovered))

# results['Inverse Filtered'] = inverse_filtered / np.max(np.abs(inverse_filtered))

# except:

# results['Inverse Filtered'] = samples_norm

# # Visualization: Time Domain Comparison

# plot_samples = 10000

# plt.figure(figsize=(16, 12))

# subplot_idx = 1

# plt.subplot(3, 3, subplot_idx)

# plt.plot(samples_norm[:plot_samples], color='blue', linewidth=0.5)

# plt.title('Original Audio Signal')

# plt.xlabel('Sample')

# plt.ylabel('Amplitude')

# plt.grid(True, alpha=0.3)

# subplot_idx += 1

# for name, audio_data in results.items():

# plt.subplot(3, 3, subplot_idx)

# plt.plot(audio_data[:plot_samples], linewidth=0.5)

# plt.title(f'{name}')

# plt.xlabel('Sample')

# plt.ylabel('Amplitude')

# plt.grid(True, alpha=0.3)

# subplot_idx += 1

# plt.tight_layout()

# plt.show()

# # Frequency Domain Analysis

# plt.figure(figsize=(16, 10))

# subplot_idx = 1

# plt.subplot(3, 3, subplot_idx)

# freq_orig = np.abs(fft(samples_norm[:plot_samples]))

# plt.plot(freq_orig[:len(freq_orig)//2], color='blue', linewidth=0.8)

# plt.title('Original - Frequency Spectrum')

# plt.xlabel('Frequency Bin')

# plt.ylabel('Magnitude')

# plt.grid(True, alpha=0.3)

# subplot_idx += 1

# for name, audio_data in results.items():

# plt.subplot(3, 3, subplot_idx)

# freq = np.abs(fft(audio_data[:plot_samples]))

# plt.plot(freq[:len(freq)//2], linewidth=0.8)

# plt.title(f'{name} - Spectrum')

# plt.xlabel('Frequency Bin')

# plt.ylabel('Magnitude')

# plt.grid(True, alpha=0.3)

# subplot_idx += 1

# plt.tight_layout()

# plt.show()

# # Impulse Responses Visualization

# plt.figure(figsize=(15, 8))

# for idx, (name, ir) in enumerate(impulse_responses.items(), 1):

# plt.subplot(2, 3, idx)

# plt.stem(ir, basefmt='b-')

# plt.title(f'{name}\nImpulse Response')

# plt.xlabel('Sample')

# plt.ylabel('Amplitude')

# plt.grid(True, alpha=0.3)

# plt.tight_layout()

# plt.show()

# Output:

# Observations:

# Five different impulse responses tested for background music suppression

# High-Pass Filter best preserves vocals while reducing low-frequency background

# Comb Filter effective for removing repetitive musical patterns

# Inverse filtering shows partial recovery but amplifies noise

# No single impulse response perfectly separates vocals from complex background music

# Convolution approach limited by overlapping frequency content between vocals and instruments

# Advanced techniques (source separation, deep learning) needed for better results