# Experiment-10

AIM: Simulate smoothing and sharpening operations on images using frequency domain filters.

Theory:

Smoothing and sharpening operations can also be performed in the frequency domain using filters such as the Gaussian filter and the Laplacian filter.

Smoothing in Frequency Domain:

To perform smoothing in the frequency domain, we apply a low-pass filter that attenuates high-frequency components. This helps to blur the image and reduce noise. One common approach is to use a Gaussian filter in the frequency domain.

Sharpening in Frequency Domain:

To perform sharpening in the frequency domain, we apply a high-pass filter that enhances high-frequency components. This amplifies the edges and fine details in the image. One common approach is to use a combination of a high-pass filter and the original image.

Program

import cv2

import numpy as np

import matplotlib.pyplot as plt

def apply_gaussian_filter(image, sigma):

# Convert image to float32 for Fourier Transform

image = np.float32(image)

# Perform Fourier Transform

frequency_domain = cv2.dft(image, flags=cv2.DFT_COMPLEX_OUTPUT)

# Shift the zero-frequency component to the center of the spectrum

shifted_frequency_domain = np.fft.fftshift(frequency_domain)

# Create Gaussian filter mask

rows, cols = image.shape

crow, ccol = rows // 2, cols // 2

mask = np.zeros((rows, cols, 2), np.float32)

for i in range(rows):

for j in range(cols):

mask[i, j] = np.exp(-((i - crow) ** 2 + (j - ccol) ** 2) / (2 * sigma ** 2))

# Apply the Gaussian filter in the frequency domain

filtered_frequency_domain = shifted_frequency_domain * mask

# Shift the zero-frequency component back to the corner

shifted_filtered_frequency_domain = np.fft.fftshift(filtered_frequency_domain)

# Perform Inverse Fourier Transform to obtain the filtered image

filtered_image = cv2.idft(shifted_filtered_frequency_domain, flags=cv2.DFT_SCALE | cv2.DFT_REAL_OUTPUT)

# Convert the filtered image back to uint8

filtered_image = np.uint8(filtered_image)

return filtered_image

def apply_sharpening_filter(image, strength):

# Convert image to float32 for Fourier Transform

image = np.float32(image)

# Perform Fourier Transform

frequency_domain = cv2.dft(image, flags=cv2.DFT_COMPLEX_OUTPUT)

# Shift the zero-frequency component to the center of the spectrum

shifted_frequency_domain = np.fft.fftshift(frequency_domain)

# Create high-pass filter mask

rows, cols = image.shape

crow, ccol = rows // 2, cols // 2

mask = np.zeros((rows, cols, 2), np.float32)

mask[crow - strength:crow + strength, ccol - strength:ccol + strength] = 1

# Apply the high-pass filter in the frequency domain

filtered_frequency_domain = shifted_frequency_domain * mask

# Shift the zero-frequency component back to the corner

shifted_filtered_frequency_domain = np.fft.fftshift(filtered_frequency_domain)

# Perform Inverse Fourier Transform to obtain the filtered image

filtered_image = cv2.idft(shifted_filtered_frequency_domain, flags=cv2.DFT_SCALE | cv2.DFT_REAL_OUTPUT)

# Convert the filtered image back to uint8

filtered_image = np.uint8(filtered_image)

return filtered_image

# Load the input image

image_path = '/content/Certificate 1698299913.jpg'

input_image = cv2.imread(image_path, 0)  # Load the image in grayscale

# Apply Gaussian filter

sigma = 20  # Adjust the value to control the amount of smoothing

smoothed_image = apply_gaussian_filter(input_image, sigma)

# Apply sharpening filter

strength = 20  # Adjust the value to control the strength of sharpening

sharpened_image = apply_sharpening_filter(input_image, strength)

# Display the original image and the filtered images side by side

combined_image = np.hstack((input_image, smoothed_image, sharpened_image))

plt.imshow(combined_image, cmap='gray')

plt.title("Original | Smoothed | Sharpened")

plt.axis('off')

plt.show()

# Save the filtered images (optional)

smoothed_path = 'smoothed_image.jpg'

sharpened_path = 'sharpened_image.jpg'

cv2.imwrite(smoothed_path, smoothed_image)

cv2.imwrite(sharpened_path, sharpened_image)

print(f"Smoothed image saved at: {smoothed_path}")

print(f"Sharpened image saved at: {sharpened_path}")

# Output

# Conclusion

- Frequency domain filtering successfully applied using Fourier Transform.

- Gaussian filter smoothed the image by attenuating high frequencies (removes noise and details).

- High-pass filter sharpened the image by preserving high frequencies (enhances edges).

- Frequency domain approach provides precise control over specific frequency components for filtering.